NYAKABINGO WOLFRAM HOST ROCK DEPTH EXTENT DRILLING

- INTRODUCTION

In the view of the ongoing resource estimation work at Nyakabingo, there is an uncertainty over how deep veins mined and mapped from underground excavations extend. Geology models have been extended to an elevation of 1000 metres (figure 2). However, it is uncertain whether the veins or the host rock extend to this elevation. It is therefore recommended that two test diamond drillholes be drilled from the surface to depths of ~700 metres (along an inclination of 70 degreed, figure 1 and 2). These test holes will add extra knowledge and confidence on the resource estimates and will therefore justify any future mine infrastructure below BV22, particularly before the current decline advance deeper.

- PLAN

The aim is two drill 1400 metres with two holes, 700 metres each as shown in the table 1 below and figures 1 and two. It should take two months or less to complete these holes. All the required funds and resources are detailed in section three.

Figure 1. Plan View of the planned drillholes at Nyakabingo (green lines).

Figure 2. Section view of the planned drillholes at Nyakabingo (green lines)

| Hole ID | Easting | Northing | RL | Target Depth | Azimuth | Inclination |
| --- | --- | --- | --- | --- | --- | --- |
| NYDD_01 | 831114.60 | 9794140.50 | 2000.00 | 700.00 | 230.00 | -70.00 |
| NYDD_02 | 830979.70 | 9794335.30 | 2006.00 | 700.00 | 230.00 | -70.00 |

Tble 1. Details of the planned drillholes

- COSTS

Assumptions are that the wholes will take 6 weeks, as learned from Ntunga drilling and all the resources are assumed to be the same as Ntunga drilling. Rates are not changed from the Energold offer.

Table 2. costs of the test drillholes.

| No | ITEM | QUANTINTY | UNIT COST$ | TOTAL COST$ |
| --- | --- | --- | --- | --- |
| 1 | Mobilisation | 2 | 3,000 | 6000 |
| 2 | Demobilisation | 2 | 3,000 | 6000 |
|  | Drill Rate (PQ) |  |  |  |
| 3 | 0 to 100m | 200 | 127 | 25400 |
|  | Drill Rate (HQ) |  |  |  |
| 4 | 101 to 200m | 200 | 125 | 25000 |
| 5 | 201 to 300m | 200 | 130.5 | 26100 |
| 6 | 301 to 400m | 200 | 136.5 | 27300 |
|  | Drill Rate (NQ) |  |  |  |
| 7 | 401 to 500 | 200 | 139.95 | 27990 |
| 8 | 501 to 600m | 200 | 146.95 | 29390 |
| 9 | 601 to 700m | 200 | 154.45 | 30890 |
| 9 | Fuel | 200 | 96 | 19200 |
| 10 | Others Charges |  |  |  |
|  | Sampling Crew |  |  |  |
| 11 | Sampling Consultant | 1 | 3500 | 3500 |
| 12 | Sampling crew | 4 | 250 | 1000 |
| 13 | Offsiders | 2 | 120 | 240 |
| 14 | Project Geologist | 1 | 8000 | 8000 |
| 15 | Resource estimation study | 0 | 30000 |  |
|  | Lab analysis |  |  |  |
|  | Lab tests and samples shipment | 600 | 40 | 24000 |
|  | Other Logistics |  |  |  |
| 16 | Car Rental | 2 | 1100 | 2200 |
| 17 | Crop/Land expropriation | 2 | 2100 | 4200 |
| 18 | Water Tank 10,000L | 3 | 750 | 2250 |
| 19 | Water Transport | 2 | 3000 | 6000 |
|  | Drill Consumables |  |  |  |
| 20 | Core Trays PQ | 50 | 6.5 | 325 |
| 21 | Core Trays HQ | 150 | 6.5 | 975 |
| 22 | Core Trays NQ | 200 | 6.5 | 1300 |
| 23 | Core blocks | 1000 | 0.51 | 510 |
| 24 | Red Chinagraph pencils - box | 1 | 1.5 | 1.5 |
| 25 | Green Chinagraph pencil - box | 1 | 1.5 | 1.5 |
| 26 | White Chinagraph pencil - box | 1 | 1.5 | 1.5 |
| 27 | Calico bags | 1000 | 0.4 | 400 |
| 28 | Permanent markers - box | 2 | 8 | 16 |
| 29 | Aluminium sample tags | 1000 | 0.15 | 150 |
| 30 | Core saw blades - green saw | 3 | 500 | 1500 |
| 31 | Core saw blades - yellow saw | 3 | 400 | 1200 |
| 32 | Flagging Tape-pink, box | 2 | 3.2 | 6.4 |
| 33 | PPE | 12 | 30 | 360 |
| 34 | Hand lense | 2 | 8 | 16 |
| 35 | Whiteboard | 1 | 30 | 30 |
| 36 | Photo rack (1*2) | 1 | 36 | 36 |
| 37 | Core tables (1*3) | 4 | 218 | 872 |
| 38 | Angle iron 3 m | 8 | 127 | 1016 |
| 39 | CRM high | 40 | 5.95 | 238 |
| 40 | CRM medium | 40 | 5.95 | 238 |
| 41 | CRM Low | 40 | 2.13 | 85.2 |
|  | Total BT |  |  | 283,938 |
| 42 | Taxes and fees Estimates (20%) |  |  | 56,788 |
|  | Sub Total |  |  | 340,726 |
| 43 | Contingency |  |  | 51,109 |
|  | Grand Total |  |  | 391,835 |

- CONCLUSION

Although the test drillholes will not be solely relied on during the resources estimates, the information to be gained will add more confidence on the resources estimates and hence support in the ongoing mine infrastructure development such as the decline, the processing plant and dams and tailing facilities. It is therefore important to drill these two holes.