### Trinity Metals Surface Water Monitoring Programme

This section presents the surface water monitoring programme for the Trinity Metals Mines, which must be implemented for the remaining life of the mines. The location of the monitoring points was identified based on the results from the hydrocensus undertaken as part of the 2024 ESIAs and they are listed in Table 1 below.

Monitoring of surface water resources must be undertaken monthly or quarterly as indicated. The monitoring programme should be reviewed by a qualified professional every six months to ensure the appropriateness of sites and sampling frequency. If the site is operated as a closed circuit in the future, with no discharges, the monitoring frequency can be reduced to quarterly.

The analytical suite listed in Table 2 is recommended for water quality analyses. Parameters listed in the table are regarded as monitoring for best practice, but additional parameters may be included as and when needed. The samples must be submitted to an accredited laboratory for analysis. Until formal stormwater management infrastructure is in place, all monitoring data must be interpreted in accordance with the following standards and guidelines:

The EHS Mining Effluent Guidelines, 2007;

WHO Guidelines for Drinking Water, 2022;

RS EAS Potable Water Specifications, 2018;

RS Effluent Standard, 2022;

RS Irrigation Use, 2019;

RS Livestock Watering, 2019; and

The EAS Standards, 2016.

Only once stormwater management infrastructure has been established and the mine site is operated as a closed circuit, with no contaminated water discharges, can the above-listed standards and guidelines be reviewed and adjusted by a qualified professional.

Table 2: Surface Water Quality Parameters of Concern

| Parameters |  |
| --- | --- |
| pH | Nitrate as N |
| Electrical conductivity | Ammonia |
| Total dissolved solids | Potassium |
| Total suspended solids | Nickel |
| Aluminum | Manganese |
| Calcium | Magnesium |
| Fluoride as F | Iron |
| Total alkalinity as CaCO3 | Copper |
| Chloride as Cl | Lead |
| Sulphate as SO4 | Sodium |
| Arsenic | - |

Table 1: Surface Water Sampling Locations

| No. | Mine | Sample ID | X_Deg | Y_Deg | Description | Monitoring Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Nyakabingo Mine | NYK D2 | 29.9718o | -1.8668o | Discharge from D2 or D4 | Monthly |
| 2 |  | NYK SW 2 | 29.220 | -1.880733 | Downstream of Trinity Nyakabingo Mine on Nyakabingo River. | Monthly |
| 3 |  | NYK SW 4 | 29.9729o | -1,9026o | Downstream of Trinity Nyakabingo Mine on Nyakabingo River, prior to confluence with the Nyabarongo River. | Monthly |
| 4 |  | NYK SW 3 | 29.9432° | -1.87917° | On the Nyabarongo River upstream of the Nyabarongo and Nyakabingo River confluence. | Monthly |
| 5 |  | NYK-TRANS 1 | 29.977923° | -1.906716° | Downstream of Trinity Nyakabingo Mine after Nyabarongo and Nyakabingo River confluence. | Monthly |
| 6 | Musha Mines | MU SW 3 | 30.358600° | -1.929550° | Downstream of Bisinia Dam and Musha Mine site, on the Nyirabigaji River. | Quarterly |
| 7 |  | MU SW 4 | 30.354167° | -1.940983° | Upstream of Bisinia Dam on Rwasama River, downstream of the Musha Mine site. | Quarterly |
| 8 |  | MU SW 2 | 30.354633° | -1.935600° | Downstream of Musha Mine site in Bisinia Dam before outlet to Nyirabigaji River. | Quarterly |
| 9 |  | NT SW 1A | 30.36615° | -1.970283° | Downstream of Ntunga Mine site, on the Gashahi River | Quarterly |
| 10 | Rutongo Mine | RT SW 10 | 30.046983° | -1.806367° | Upstream of Nyamyumba Mine, on the Sanzare River | Monthly |
| 11 |  | RT SW 16 | 30.052617° | -1.806317° | Downstream of Nyamyumba Mine on the Sanzare River | Monthly |
| 12 |  | RT SW 6 | 30.050850° | -1.792950° | Upstream of Gasambya Mine, on the Kivomo River | Monthly |
| 13 |  | RT SW 15 | 30.058100° | -1.795933° | Downstream of Gasambya Mine on the Kivomo River | Monthly |
| 14 |  | RT SW 14A | 30.050633° | -1.769750° | Upstream of Karambo Mine on the Nyirabukingure River | Monthly |
| 15 |  | RT SW 21 | 30.055733° | -1.768783° | Downstream of Karambo Mine on the Nyirabukingure River | Monthly |
| 16 |  | RT SW 17 | 30.082167° | -1.790317° | Upstream of Mahaza Mine on the Rusine River | Quarterly |
| 17 |  | RT SW 13 | 30.080850° | -1.795467° | Downstream of Mahaza Mine on the Rusine River | Quarterly |
| 18 |  | RT SW 5 | 30.085367° | -1.804867° | On the Rusine River. To monitor the cumulative impact from upstream mines. | Quarterly |
| 19 |  | RT SW-MASO 1 | 30.044914° | -1.824729° | Upstream of the Masoro Mine on the Gatiti River | Quarterly |
| 20 |  | RT SW-MASO 2 | 30.047392 | -1.829757 | Downstream of the Masoro Mine on the Gatiti River | Quarterly |
| 21 |  | RT SW 9 | 30.045467° | -1.833217° | Downstream of the Masoro Mine just after the Mulindi River and Gatiti River confluence | Quarterly |
| 22 |  | RT SW 1 | 30.014650° | -1.778217° | Upstream of Gisanze Mine on the Nyamukamba River and the Bahimbiguye confluence | Quarterly |
| 23 |  | RT SW 4 | 30.019883° | -1.772933° | Downstream of Gisanze Mine on the Rusine River | Quarterly |

### Groundwater Monitoring

Initially, for the next six months, monitoring (quality and quantity) must be undertaken monthly until a longer-term baseline has been established. The monitoring plan must be reviewed after six months to ensure the appropriateness of sites and sampling frequency during operations. When the baseline is well established, the monitoring plan can be reviewed every three years. The minimum list of parameters to be analysed is included in Table 4. The list of parameters will change over time as new parameters are added, and some are removed based on the results of a continuous monitoring programme. These borehole locations are shown in Figure 6-2. The monitoring standards that need to be considered are the same as those outlined under the surface water monitoring programme.

Table 4: Groundwater Quality Parameters of Concern

| Parameters | Parameters |
| --- | --- |
| pH | Nitrate as N |
| Electrical conductivity | Ammonia |
| Total dissolved solids | Potassium |
| Total suspended solids | Nickel |
| Aluminum | Manganese |
| Calcium | Magnesium |
| Fluoride as F | Iron |
| Total alkalinity as CaCO3 | Copper |
| Chloride as Cl | Lead |
| Sulphate as SO4 | Sodium |
| Arsenic | - |

Table 3: Monitoring Borehole Locations

| No | Mine | Borehole ID | Longitude | Latitude | Objective/Target | Description | Monitoring Frequency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Nyakabingo | NBH1 | 29.97195 | -1.87597 | The overall impact of mining | Located approximately 1 km south of Nyakabingo settling dams | Monthly for 1st 6 months |
| 2 |  | NBH2 | 29.97231 | -1.86707 | The overall impact of mining | Located below Nyakabingo settling dams | Monthly for 1st 6 months |
| 3 | Musha Mines | MBH1 | 30.34025 | -1.93084 | Background monitoring | Located approximately 300 m southwest of the Musha Mine offices. | Monthly for 1st 6 months |
| 4 |  | MBH2 | 30.3483 | -1.93192 | The potential impact of mining | Located at the entrance of the new Heza decline shaft. | Monthly for 1st 6 months |
| 5 |  | MBH3 | 30.34004 | -1.93084 | Potential Impact of Settling Dam | Located approximately 300 m southwest of the Musha Mine offices. | Monthly for 1st 6 months |
| 6 |  | MBH4 | 30.33997 | -1.92436 | Baseline data | Located approximately 400 m northeast of the Musha Mine offices. | Monthly for 1st 6 months |
| 7 |  | NBH1 | 30.36132 | -1.96264 | The potential impact of mining | Located approximately 100 m west of the lower adit. | Monthly for 1st 6 months |
| 8 |  | NBH2 | 30.36246 | -1.96351 | The potential impact of mining | Located approximately 120 m south of the lower adit. | Monthly for 1st 6 months |
| 9 | Rutongo Mines | RBH1 | 30.04579 | -1.82553 | The overall impact of mining | Located at the Masoro Mine | Monthly for 1st 6 months |
| 10 |  | RBH2 | 30.0461 | -1.82988 | The overall impact of mining | Downstream of Masoro Mine | Monthly for 1st 6 months |
| 11 |  | RBH3 | 30.056 | -1.7945 | The overall impact of mining | Located east of Gasambya Mine along the drainage line | Monthly for 1st 6 months |
| 12 |  | RBH4 | 30.05309 | -1.80693 | The overall impact of mining | Located at the Nyamyumba Mine | Monthly for 1st 6 months |
| 13 |  | RBH5 | 30.02046 | -1.77255 | Baseline data | Located approximately 2.4 km northeast of Rutongo Hospital | Monthly for 1st 6 months |
| 14 |  | RBH6 | 30.05726 | -1.7688 | Water supply and background monitoring | Located <100 m northwest of Karambo B Primary School | Monthly for 1st 6 months |
| 15 |  | RBH8 | 30.08207 | -1.79405 | The overall impact of mining | Downstream of Mahaza Mine next to the road | Monthly for 1st 6 months |

A-1	Surface Water Sample Collection Field Form

|  |  |  |  |  | Mine: |  |
| --- | --- | --- | --- | --- | --- | --- |
| Month/Year: (e.g 01/25) |  |  |  |  | Collector name: |  |
| Weather: |  |  |  |  | Collector signature: |  |
| Date | Time | Location (e.g. mine site, upstream or downstream) | Site Code | Comments (Note any conditions on site, other impacts, etc) |  |

A-2	Groundwater Sample Collection Field Form

|  |  |  |  |  | Mine: |
| --- | --- | --- | --- | --- | --- |
| Month/Year: (e.g 01/25) |  |  |  |  | Collector name: |
| Weather: |  |  |  |  | Collector signature: |
| Date | Time | Location (e.g. mine site) | Site Code | GW Depth | Comments (Note any conditions on site, other impacts, etc) |