EXPRESSION OF INTEREST FOR PROVISION OF DIAMOND DRILLING SERVICES FOR DEPTH EXTENSION OF VEINS AT NYAKABINGO TO TRINITY METALS LTD

Trinity Nyakabingo Mine Ltd, invites Expressions of Interest (EOI) from qualified and experienced Rwandan drilling contractor to provide locally sourced Diamond Drilling Services for the purpose of defining the extension of veins at Nyakabingo Mine, Western Province, Rwanda. The program aims to evaluate vein depths, vein widths and sampling for Mineral Resource Estimation.

Scope of Work for Diamond Drilling at Nyakabingo Mine:

The scope of work for the Diamond Drilling work includes but is not limited to the following:

- Mobilization and Site Establishment

  - Mobilization of a diamond drilling rig located in Rwanda and support the transport of equipment to site

  - Preparation of safe drill platforms on tailings areas

  - Compliance with site-specific Health, Safety, and Environmental (HSE) requirements

  - Coordination with Trinity Nyakabingo technical personnel

- Diamond Drilling Operations

  - Demonstrated experienced diamond drilling operators and site managers

  - 1400 meters total from 2 boreholes each at 700m

  - Critical spares and skilled team for daily operating and use of the drill rig

  - Execution of a planned grid-based drilling program

  - Continuous sampling using diamond coring techniques ensuring protocols in place to ensure integrity of the cased sample

  - Drilling through soft, loose, and variably compacted tailings

  - Core size required is PQ from 0-100 meters HQ from 101-400 meters and NQ beyond that.

  - Accurate recording of penetration rates, recovery, and ground conditions

  - Accurate recording of meters drilling.

  - Understanding of oriented core and geotechnical practices.

- Sampling and Core Handling

  - Delivery of continuous, relatively undisturbed core samples

  - Proper containment using liners, sleeves, or core boxes

  - Collaboration with site geologists while logging and sampling

  - Prevention of sample loss, mixing, or contamination

- Data Recording and Reporting

  - Daily drilling reports including depth, recovery, and operational parameters

  - Borehole collar survey and final depth confirmation

  - Provision of as-drilled logs and coordinates

  - Compliance with Trinity Nyakabingo QA/QC procedures

- Demobilization and Rehabilitation

  - Removal of equipment and restoration of work areas

  - Safe sealing/abandonment of boreholes

  - Environmental compliance in line with international and local Rwanda regulatory standards.

Eligibility and Competency Requirements

Interested parties must:

- Proven experience in diamond drilling for resource estimation tailings, alluvial, or soft-ground environments

- Availability of suitable diamond drilling rigs and tooling

- Experienced operators and supervisory personnel

- Strong safety culture and documented HSE systems

- Ability to mobilize and operate within Rwanda

All interested parties should submit an Expression of Interest in accordance with the below requirements complete with supporting documents.

- Application Cover Letter introducing your Company/Individual expressing interest, availability, and a brief business and experience profile including qualifications and experience

- Technical specifications of proposed drilling equipment

- CVs of proposed personnel demonstrate diamond drilling experience for Mineral Resource estimation purposes.

- Competent Person according to the reporting codes.

- Summary of qualifications and relevant experience.

- Indicative timeframe and approach.

- Details of at least five similar projects undertaken in the last five years.

- Certificate of domestic company registration issued by RDB.

- VAT Registration.

- Valid income tax clearance issued by RRA

Submission to be forwarded to tender@trinity-metals.com by not later than 20th March 2026 at 11:00AM Local time. Further discussion will be undertaken with selected respondents to prove the competence of the bidder and thereafter an invitation to tender will be provided to the bidder.